������������������������������������������������������������ͻ

�                                                            �

�                ECRLINK v1.10 beta test                     �

�                                                            �

������������������������������������������������������������ͼ

The EcrLink system has been designed to upload the

programming and sales information from certain models of SHARP

cash registers, save this information on disk and download to the

cash register again if required.

Anyone who has had the misfortune to lose this information

and has spent, up to a few weeks in some cases, time restoring

this information, will, I'm sure, appreciate this system. You do

not need any fancy boards installed in your register as the

system works through the service port built into these machines,

usually used for transferring from one register to another.

It would be nice to be able to write that this program can

be used with any of the Sharp models but, unfortunately, Sharp

has changed the comms protocols on some ranges and is reluctant

to release the information as if it is some great secret.

Anyhow, this program has been tested on desktop and laptop

machines, from XT's to 486's ( 286 or better recommended ) and

seems to work OK. Cash registers tested are the ER-3100, ER-3110

and the ER-2910, others with the same protocol should work OK.

At the moment, a RS-232C level convertor, the ER-52LC, ER-67LC

or equivalent is required because of the different voltage levels

on the PC and Register serial ports. I would like to incorporate

a level convertor in the actual transfer cable, probably powered

by the PC's port and I would appreciate any info on designing this.

The special cable with the correct DIN type plug for connecting

to the register comes complete with the level convertor.

PC to ECR level convertor cable

PC 9-Pin           ER-52LC/67LC 25-Pin

25-Pin in brackets

TX    3 (2) ��������������������� 3   RD

RX    2 (3) ��������������������� 2   SD

CTS   8 (5) �������������������� 20   ER

�

DSR   6 (6) �����������

SG    5 (7) ��������������������� 7  GND

DTR   4 (20) �������������������� 5   CS

�

����������� 6   DR

�

����������� 8   CD

I would also be most grateful to get hold of upload/download

protocol for the ER-4100 and ER-4110 as these are popular units,

with large memories, but whose port handshaking is different.

Please send your comments and/or suggestions as to how you

find the system, improvements that could be made etc. to any of

the addresses below.

��������������������������������������������������ͻ

�  Phil Webb                                       �

��������������������������������������������������Ķ

�  P.O. Box 34349, Jeppestown,                     �

�  2043, Republic of South Africa                  �

��������������������������������������������������Ķ

�  Beltel Mail Number: 802240                      �

��������������������������������������������������Ķ

�  Internet Email: phil.webb@vcbbs.proxima.alt.za  �

��������������������������������������������������ͼ