This document is designed to assist Digiboard customers in converting

custom device drivers from the PC/8e with a fixed memory window to the

newer version with a memory window interface.

The procedures for loading the BIOS and FEP are discussed along with how

to access channel buffers. Not all cases where a driver has to access

memory are discussed, but it is hoped that these examples will illustrate

how to interface to the Digiboard with a memory window interface.

In the discussion to follow the following notation will be used:

devb[i]    The ith byte of board I/O space

memb[i]    The 8-bit byte of board memory at board relative address i

as viewed by the host.

memw[i]    The 16-bit word of board memory at board relative address i

as viewed by the host.

In fixed memory window products memory was enabled by placing 0x02 in the

io port base register.

devb[0] = 0x02 ;

Memory window products enable a single page of memory by selecting the

memory page in the lower bits of the second io port and a 1 in the upper most

bit to enable memory.

devb[1] = 0x80 ;        enables window zero

devb[1] = 0x87 ;        enables window seven

Loading the BIOS

The BIOS is loaded into the upper region of memory(specifically at

0xff800).

For the old board this is done with the following code:

devb[0] = 0x06 ;                /* hold reset and enable memory */

mem_ptr = board_addr+(0xff800-membase);

for(i = 0; i < n; i++)

{

*mem_ptr++ = bios[i];

}

board_addr + 0xC00 = 0;         /* clear bios start flag */

devb[0] = 0x02 ;                /* release reset to start BIOS */

Where:

board_addr is the base address of the board as seen by the host

mem_ptr is a pointer to board memory

membase is the base address of dual ported memory. (0xf0000 for

64K)

bios[] is a char array containing the BIOS code.

For the new board this is done with the following code:

devb[0] = 0x04 ;                /* board held in reset */

devb[1] = 0x87 ;                /* Select page 7 and enable memory

*/

mem_ptr = board_addr + 0x1800 ;

for(i = 0; i != n; i++)

{

*mem_ptr++ = bios[i];

}

devb[1] = 0x80 ;                /* point to window 0 for global

data */

board_addr + 0xC00 = 0 ;        /* clear bios start flag */

devb[0] = 0x00 ;                /* release reset to start bios */

Loading the FEP

The FEP code is loaded at location 0x2000 as seen by the PC/Xe

For the old board this is done as follows:

mem_ptr = board_addr + 0x2000 ;

for(i = 0; i < n; i++)

{

*mem_ptr++ = fep[i];

}

board_addr + 0xc40 = 02 ;       /* Command */

board_addr + 0xc42 = 0x200 ;    /* From */

board_addr + 0xc44 = 0 ;

board_addr + 0xc46 = 0x200 ;    /* To */

board_addr + 0xc48 = 0;

board_addr + 0xc4a = 0x2000 ;   /* FEP/OS size */

devb[0] = 0x0a ;                /* Interrupt to BIOS */

devb[0] = 0x02 ;

Where:

fep[] is a char array containing the FEP code.

For the new board the procedure is as follows:

devb[1] = 0x81 ;                /* point at window 1 */

mem_ptr = board_addr ;

for( i = 0; i != n; i++)

{

*fep_ptr++ = fep[i] ;

}

devb[1] = 0x80 ;                /* point at window 0 */

board_addr + 0xc40 = 02 ;       /* Command */

board_addr + 0xc42 = 0x200 ;    /* From */

board_addr + 0xc44 = 0 ;

board_addr + 0xc46 = 0x200 ;    /* To */

board_addr + 0xc48 = 0;

board_addr + 0xc4a = 0x2000 ;   /* FEP/OS size */

devb[0] = 0x08 ;                /* Interrupt to BIOS */

devb[0] = 0x00 ;

Accessing Channel Buffers:

When using the old board, the method to access a channel receive buffer

is as follows:

/* get board address from channel structure receive segment */

chan_rcv = board_addr + (chan_ptr->rseg - base_seg) << 4 ;

/* get character from head of buffer */

while (chan_ptr->rout != chan_ptr->rin)

{

rcv_char = chan_rcv + chan_ptr->rout ;

chan_ptr->rout = (chan_ptr->rout + 1) & chan_ptr->rmax ;

}

Where:

chan_rcv is the start of the channel receive buffer.

chan_ptr is a pointer to the channel structure.

base_seg is the memory segment for the base of dual ported

memory as seen by teh PC/Xe.

The procedure for the new board is as follows:

/* enable window 0 */

devb[1] = 0x80 ;

rin_sav = chan_ptr->rin ;

rout_sav = chan_ptr->rout ;

rmax_sav = chan_ptr->rmax ;

reg_sav = chan_ptr->rseg ;

while(rin_sav != rout_sav)

{

/* calcluate offet of data from window base */

win_offset=((rseg_sav <<4) & 0x7fff) + rout_sav ;

/* select correct window */

devb[1] = 0x80 | (rseg_sav >>11)) ;

/* read in character */

rcv_char = board_addr + win_offset ;

/* update local copy of buffer pointer */

rout_sav = (rout_sav + 1) & rmax_sav ;

}

/* reselect base window */

devb[1] = 0x80 ;

chan_ptr->rout = rout_sav ; /* update out pointer */

Where:

rin_sav is a local copy of the receive input pointer

rout_sav is a local copy of the receive output pointer

rmax_sav is a local copy of the buffer size mask

rseg_sav is a local copy of the buffer segment as seen by the

PC/Xe

win_offset is the offset into the current window in bytes