SLIP package	Page 1	March 1, 1993

*******

8-24-94

Modified by John Mudge for the SLIP_DOS package...but still requires UMinn

registration!

*******

What is SLIP?

How Does It Work?

What is the SLIP package?

SLIP.BAT

UMSLIP.COM

TERMIN.COM

PHONE.EXE

The HELP system (F1 Key)

Hot Keys

Cross-References (Hypertext)

Mouse

Previous Key (Alt-F1)

Index

Contents

Help-with-Help

The Primary Screen

The Menu Bar

Selecting from the Menu Bar

The System (Alt-Space)

Actions (Alt-A)

Setup (Alt-S)

Server Window

Time Display

The Lower Status Bar

Active Keys display

Context-Sensitive display

Transaction Log Window

Command-line operations

PHONE DIAL

PHONE HANGUP

PHONE FORCE <command>

PHONE WRITE

PHONE ?

Scripts

Built-in scripts

How to make custom scripts

PHONE WRITE

PHONE.CMD

The SLIPScript Language

SLIPScript Elements

SLIPScript commands

Writing a script

Debugging a script

Let us know about your scripts.

What is SLIP?

SLIP stands for Serial Line Internet Protocol, and is a way to

establish a network connection through a modem.  What this means

is that programs like POPMail, Gopher and NCSA Telnet will operate

over telephone lines and through your modem, as if connected to the

network by a network card.

Normal network connections are usually very fast (on the order of 10

million bits per second, as compared with 10 thousand bits per

second with the fastest modem), so applications running over a SLIP

connection may seem to run somewhat more slowly than they would

on a hardwired network connection.

However, there are many advantages to using networked

applications rather than standard modem communication programs

such as Procomm.  For one, you can take advantage of client-server

protocols such as Gopher or POP, which allow your computer to do

some of the computing work, and to store the files received on your

hard drive rather than on a remote host computer.  Another

advantage is the ability to run multiple sessions:  programs like NCSA

Telnet will allow you to connect to multiple hosts at one time, and to

transfer files in the background while working in another window.

In order for this to work, your local network must have a "SLIP

server".  A SLIP server is a computer which monitors incoming

modems for SLIP connections and translates the modem signals into

standard TCP/IP network signals.  You may also have some form of

security system controlling who can connect to a SLIP server and

receive an IP address.  You should check with your local network

administrator to see if SLIP is available.

How Does It Work?

Developers of network applications often design their software to use

the Packet Driver Specification.  A packet driver is a small piece of

software that handles communications between your computer and

the outside world.  A packet driver communicates with an ethernet

board, a Token Ring card, a serial communications port, or some

other piece of hardware that is connected to the network.  It

provides the "glue" that allows the network application to

communicate properly with the network.  Applications written to the

Packet Driver Specification do not need to know how to communicate

with ever possible network adapter; they just need tonow how to

address the packet driver.  Figure 1 shows how this works.

Figure 1.  How network packet drivers work.

The UMSLIP packet driver is written specially for SLIP

communications, and uses your serial port instead of a network

adapter card to make a network connection.

Figure 2.  How UMSLIP makes a network connection

Network applications such as MINUET talk only to the

packet driver interface to get their information.  The UMSLIP packet

driver replaces the packet driver normally used with a network

adapter card, and  sends the data out the serial port of your

computer instead (Figure 2).  The network application can't tell the

difference:  it's only talking to the interface.  So all your network

applications function as if they are physically connected to the

network.

What is the SLIP package?

Everything that's necessary to make a SLIP connection can be found

together in the SLIP_DOS.ZIP compressed file.  These include:

SLIP.BAT

A convenient batch file which makes it easy to use SLIP.

UMSLIP.COM

The SLIP packet driver which translates program network signals

into modem signals.

TERMIN.COM

The unaltered NCSA TERMIN.COM program for removing

UMSLIP.COM from memory.

PHONE.EXE

The program which dials the telephone and establishes a SLIP

connection.

Documentation

PHONE.DOC is this file

PHONE.HLP is the on-line help database for PHONE.EXE

UMSLIP.DOC is a help file for UMSLIP.COM.

SLIP.BAT

SLIP.BAT is a batch file that automates SLIP.  Simply type "SLIP"

to initiate a SLIP connection.

UMSLIP.COM

The PHONE program needs the UMSLIP packet driver in order to talk

to the modem.

UMSLIP should be run from your AUTOEXEC.BAT file to ensure that it is

always ready.

UMSLIP uses about 6k of RAM and can be run under DesqView loaded high.

TERMIN.COM

TERMIN.COM will remove the UMSLIP.COM packet driver from

memory.

PHONE.EXE

PHONE establishes a connection to the SLIP server or terminates an

active connection.  It does this by sending commands to your modem

to dial the phone, and then sending your login information to

validate the connection with the SLIP server.  The UMSLIP.COM

program must be run before PHONE.EXE.

The HELP system (F1 Key)

To access PHONE's Help System, press the F1 key.  A window will

open containing information about the menu item or dialog box field

that is currently in the foreground.  This is known as "context-

sensitive" help.

You can browse through the help text using the

Description	Key

up arrow		-

down arrow 	/

Page Up		PgUp

Page Down		PgDn

keys on the keypad.

When you are finished using the help, press the Escape (Esc) key.

Hot Keys

Some menu commands are considered to be so important that they

need to be available with a quick keypress short-cut:  a "hot key"

that goes straight to that command.  Hot keys for PHONE are shown

below.

Hot-Key	Command

F1		Help

F2		Dial

F3		Hang up

F5		Exit

Alt-X	Exit

Cross-References (Hypertext)

Some help messages contain Cross-References to other topics.   Cross-

References are displayed in a different color from normal help text.

One cross-reference is always highlighted.  If you press Enter, the

help message associated with the cross-reference is displayed.  If

there are several cross-references within a help message, you can

press the Tab or Shift-Tab keys to move the highlight to the next or

previous cross-reference.

Mouse

If your computer has a mouse, move the cursor over a cross-

reference and click the left mouse button twice to select it.  Double

Clicking (pressing the left mouse button twice in rapid succession)

has the same effect as pressing the Enter key.

Previous Key (Alt-F1)

Press the PREVIOUS button (Alt-F1) to "back up" one page to the

previously-selected HELP topic.

Index

An alphabetical listing of every on-line help entry.

Contents

Help entries organized by subject and category.

Help-with-Help

You can use PHONE's on-line Help from the Help menu or via hot

keys.

Wherever you are in PHONE, you can press F1 to see a help

screen about the menu command or dialog-box item that's

currently active.

To learn more about using on-line Help, use the arrow keys

to move the cursor to one of the Help keywords; then press ENTER.

You can also press TAB to jump to the next Help keyword,  and then

press ENTER.

If you're using a mouse, just click the Help keyword you're

interested in.

The Primary Screen

The Primary Phone Screen consists of four parts:  a top Menu Bar, a

message space below, a Transaction Log window where the progress

of the connection can be observed, and a lower Status Bar.

The Menu Bar

The Menu Bar consists of the primary groups of PHONE commands

available on pull-down menus.  You can open these menus by

selecting a given Menu Bar item with the mouse (by positioning the

cursor over a given item and clicking the left button), or by pressing

keys for those who don't use a mouse.

Selecting from the Menu Bar

There are three ways to select a menu command:

F10

Press F10, then use the left and right arrow keys to select the

appropriate menu and use the the up and down arrow keys to select

a command.  Press Enter to choose the selected command.

Alt

Press Alt and the highlighted letter of the menu you want (such as

Alt-F for the File menu), use the up and down arrow keys to select a

command, then press Enter to choose that command.

You can also press the highlighted letter of a menu name or

command instead of using the arrow keys.

Mouse

Click on the title of the menu you want to pull down, then click on

the menu command you want to choose.

The highlighted menu title is the currently selected menu.

Menu commands are followed by an ellipsis (...), an arrow, or a hot

key.  An ellipsis indicates that selecting that option will bring up a

dialog box, as opposed to immediately initiating a program action.

The System (Alt-Space)

The System appears on the far left of the menu bar and is

represented by the = symbol.  Alt-Spacebar is a keyboard shortcut

that you can use at any time to pull down this menu.

The = menu contains the "About" item.  Some other programs (like

MINUET) will provide some utilities under this menu.

About

When you choose the About command from the System menu, a

dialog box appears, showing copyright and version information.

Press ESC, click OK, or click Cancel to close the box.

Actions (Alt-A)

Phone can either Dial (F2) to establish a SLIP connection, or

Hang Up (F3) to break the connection and clear the phone line.

Finally, you can Exit (Alt-X) from the program.

Dial (F2)

Once you have used the Setup options to configure your modem and

to enter  your user name and password, the DIAL command will

place the call and establishing the SLIP connection.

You can watch the lower Transaction Log window to observe the

progress of the connection.

After you have configured the DIAL menu, you can also type the

command PHONE DIAL from DOS.  This will run PHONE and attempt

to establish the SLIP connection with no further commands on your

part.

If the program fails to establish the connection for some reason, it

will return an ERROR CODE, it can be used in DOS batch files (such as the

SLIP.BAT file, see above) to decide whether to proceed to the next

command in a batch file.

You can begin dialing by using the mouse to click on Actions:Dial,  by

typing Alt-A then D, or by pressing the F2 key.

Hangup (F3)

If your modem is already connected to a SLIP server or any other

service, HANGUP will disconnect it and hang up the phone.  You can

also do this by typing the command PHONE HANGUP from DOS.

You can hang up by using the mouse to click on Actions:Hangup, by

typing Alt-A and then H, or by pressing the F3 key.

Quit (Alt-X)

Use the Exit command when you are done using PHONE.

Note that this does not hang up the phone or break your SLIP

connection (see the Actions:Hangup command).  It simply quits from

the PHONE.EXE program.

You can quit PHONE by using the mouse to click on Actions:Exit, by

typing Alt-A and then X, or by typing Alt-X.

Setup (Alt-S)

Before you can establish a SLIP connection, you need to configure

both your Modem parameters (F4) (Baud Rate, Word Length, etc.)

and your SLIP parameters (F5) (Username, Password, etc.)

Modem (F4)

The settings in this dialog box are used to tell your computer how to

talk to your modem.  The speed of the modem, where it's connected,

and what kind of modem it is are among the characteristics which

need to be set so that your computer and modem can communicate.

While this window may look complex, you should need to set only a

few of the options.  You will need to select the Baud Rate, Com Port,

and Modem List entries will need to be set, but the Advanced entries

are likely to be fine the way they are.

Baud Rate

The Baud Rate is the speed at which modems and computers talk to

each other.

If your modem does not feature data compression, click on the

highest baud rate at which your modem can communicate.  If your

modem does feature data compression, click on a rate 2 to 4 times

higher than the highest baud rate at which your modem can

communicate.  Examples:

If you have a Hayes (TM) SmartModem 1200, a modem that does not

do data compression, click on 1200.

If you have a Multi-Tech (TM) MultiModem II, which is a 9600 baud

modem with data compression, click on 19,200 or 38,400.

If you choose a baud rate higher than that at which your modem can

communicate, you'll also have to check the boxes labeled CTS and RTS

Handshaking (see below).  This prevents your modem from

overwhelming your computer with data.

Data Errors

Your modem may be able to detect transmission errors (caused by

static on the line, for instance) and correct them.  It corrects the

error by signaling the remote computer that an error occurred, and

requesting that the block of data be sent again.  If errors occur, your

final Baud Rate may seem to be much lower than that set.

Transmission Bottlenecks

In any SLIP connection, several devices are transferring information

in a "bucket brigade".  No matter the settings you use, the line can

only go as fast as the slowest device.  For example, you might set

your computer's baud rate to talk to its modem at 9600 baud.

However, the modem itself might only be a 2400 baud modem.

Therefore,  you'll only see 2400 baud transfers on your screen.

Data Compression

Finally, a factor which speeds things up instead of slowing them

down.  Data compression is a means by which slow modems can send

data faster than would normally be possible.  If your modem

supports data compression (and this will be clearly indicated on the

box at time of purchase, as it is a strong selling point), it will look at

the data that it receives to see if there are any ways in which the

data can be reduced in size.  For example, if it receives the letters

"aaaaaaaaa", it might compress them into "9*a".  Of course, the

modem on the other end of the telephone must know how to de-

compress this data correctly.

COM Ports

Most IBM-compatible computers come with four possible

"Communications Ports".  Usually only one or two of these is built in

and others must be added separately.  These built in ports are COM1

and COM2.  If you are not sure which port your modem is connected

to, start with COM1, then try COM2.  If nothing works, you may have

to seek assistance in determining your setup.

COM 1

The most commonly-used Communications port is COM1.  Click on the

COM1 button if your modem is plugged into the COM1 port.  You'll

have to figure out what plug-in on your computer is the COM1 port

as they are rarely labeled.

If your computer has COM1 and COM2 already occupied, or you have

some special cards that use the COM interrupts, you may have your

COM ports at a non-standard address (see below).

COM 2

COM2 is also frequently used for communications.  If you have a

mouse connected to COM1, you will want to use COM2 for

communications.

COM 3 and COM 4

COM3 and COM 4 are not as commonly used as COM1 or COM2.  Part

of the reason for this is that these ports use the same Interrupt

signals as COM1 and COM 2 (Interrupts 4 and 3, respectively).   For

example, if you have a device on COM1, you cannot put a device on

COM3, because whenever the computer sends a signal to COM1 both

devices will try to answer at once.

Nonstandard Ports

For individuals who have filled their standard (COM1 through COM 4)

ports, several nonstandard ports are available, but not

recommended:

COM5 is actually COM3, but using interrupt level 2.

COM6 is actually COM4, but using interrupt level 2.

COM7 is actually COM3, but using interrupt level 5.

COM8 is actually COM4, but using interrupt level 5.

Real techweenie stuff

Below is a chart of I/O addresses and Interrupt levels.  Most users

won't have to worry about this.  If you have your COM  port at a

non-standard I/O address or interrupt level, then you'll have to

consult this chart to figure out what COM port your serial port uses.

Unfortunately there are two sets of these numbers, one for non-

MicroChannel PC's and one for MicroChannel PC's.  MicroChannel PC's

are mainly IBM PS/2 models 50 and above.

MicroChannel				ISA/EISA

PORT	IRQ	I/O Address	PORT 	IRQ	I/O Address

com1	4 	03f8 		com1	4	03f8

com2 	3	02f8 		com2	3	02f8

com3	3	3220		com3	4	03e8

com4	3	3228		com4	3	02e8

com5 	3	4220		com5 	2	03e8

com6 	3 	4228		com6	2	02e8

com7 	3 	5220		com7	5	03e8

com8	3 	5228		com8	5	02e8

Modems

It's important to know what kind of modem you are using when

establishing a SLIP connection.  The Modem List displays built-in

scripts for operating Hayes, ADI-100, and other kinds of modems.  It

is possible to build customized scripts if your modem is not listed

(see SCRIPTS below).

HAYES

As the standard by which other modem command sets are measured,

the Hayes script ought  to be compatible with any modem.  Of course

it isn't, which is why customized scripts are needed (see SCRIPTS

below).

ADI

The University of Minnesota's internal telephone system operates

over a digital network, allowing both voice and computer data to be

transmitted over the same phone line, and allowing for the routing of

computer data into the SLIP server without using a standard modem.

Instead, offices at the University which do not yet have a standard

network connection can order an ADI-100 transceiver with MKO

(Machine Keyboard Originate) enabled.  For information about this

procedure, U of M staff can contact the Telecommunications

Department at 6-7800.

The ADI script is to be used with these ADI-100 connections.

Other

We will add other modem scripts to this list as time for testing

allows.  If you create a customized script which you think would be

useful to others, please feel free to send it to us at

slip@boombox.micro.umn.edu

Advanced Configuration

Parity

Parity Checking is the means by which your computer can make sure

that transmitted data is getting through correctly.  Different hosts

need different parity settings.

Odd Parity means that the parity of all data words received should

be set odd.

Even Parity means that the parity of all data words received should

be set even.

Mark Parity means that no error checking is conducted, but the

parity bit is always set to one.  Nobody uses it.  There is no truth to

the rumor that Mark Parity invented the first modem.

Even fewer people use Space Parity than Mark parity.  Space parity

means no error checking is conducted, but the parity bit is always

set to zero.

Stop Bits

Stop Bits are used to separate words of data.  The more stop bits

employed, the longer the pause between data words.  This used to be

an issue when data transmission technologies were new, but

nowadays stop bits are not quite as important.  One is usually

sufficient.

Two stop bits are usually more than are necessary.

Handshaking

Handshaking is the means by which two computers tell each other

when they are able and unable to accept further data.  There are two

ways to do this: Software and Hardware.

Software Handshaking

Software handshaking controls data flow by sending a signal called

"XON" for "Transmit On" (usually a control-Q), or "XOFF" for "Transmit

Off" (control-S).

The problem with this method is that if you send an XON or XOFF

signal inadvertently (for instance, by sending a binary file which

happens to contain a control-S character), your computer will

interpret this as an XOFF and stop sending.

So for SLIP transmissions, it usually best to use either NO

handshaking, or to use the other method of handshaking, Hardware

Handshaking.

Hardware Handshaking

Hardware handshaking depends on the actual pins connecting your

computer  to your modem to control data flow.  If you are using a

cable which does not have wires connecting pins number 4 and 5 (on

a 25-pin cable or 7 and 8 on a 9-pin cable) , hardware handshaking

will not function.  The two signals carried on these pins are Clear-To-

Send CTS and Request-To-Send RTS.

CTS, or Clear-To-Send Handshaking, sets pin 5 of your serial port

either ON or OFF depending on whether or not your computer is

ready to receive more data from your modem.

RTS, or Request-To-Send Handshaking, sets pin 4 of your serial port

either ON or OFF depending on whether or not your computer is

ready to send more data to your modem.

If you are using a modem with data compression, set CTS and RTS

handshaking ON, otherwise leave it OFF.

Signals and Cables

There are nine signals which comprise the serial communication

signals from your computer to its modem (see table below).  These

signals are carried between your computer and modem by a modem

cable.  While nine signals are available, it is possible to skimp and get

by on only four of the nine signals:  SD, RD, DSR and DTR.  Sometimes

cable manufacturers will produce inexpensive cables by only running

four wires carrying these signals between the ends of the cable.

SLIP communications are much more demanding, and require all

nine signals to be present.  Be sure to use serial cables with all signal

lines in place.  One way to be sure is to purchase "flat" or "ribbon"

cable, which carries all 25 lines in a flexible plastic ribbon.

Table.  Serial communication signals and pins.

Signal Description	Code		25 Pin #		9 Pin #

Transmitted Data	SD		2			3

Received Data		RD		3			2

Request to Send		RTS		4			7

Clear to Send		CTS		5			8

Data Set Ready		DSR		6			6

Signal Ground		SG		7			5

Carrirer Detect		CD		8			1

Data Terminal Ready	DTR		20			4

Ring Indicator		RI		22			9

Plugs

There are two kinds of plug to be found on modem cables.  The first

kind, 25-pin, is a wide, flat plug with two rows of pins.  Only nine of

these signals are used for serial communications, even if all 25 pins

are present (sometimes the unused pins are left out of the plug

itself).

The second kind of plug is a 9-pin connector.  It has one pin for

every signal necessary for serial communications.  The table below

indicates which pins carry which signal on each kind of plug.

Note that hardware handshaking will only work if the cable between

your modem and PC has the proper wires carrying Request to Send

and Clear toSend (pins 4 and 5 on a 25-pin connector, 7 and 8 on a 9-

pin connector) running from end to end.  If your modem cable does

not carry all nine signals, you will need to purchase a cable with all

lines in place.

Dialing

Touch tone dialing is present when you can place a call by dialing

with musical tones.  Sometimes you might think you have touch tone

because you have a fancy telephone which has buttons; but if it

doesn't actually complete your call until doing a lot of clicking, it's

likely converting your button-pushes into pulse signals.

Determine which type of telephone service you have, and select

either Tone or Pulse .  When in doubt, use Pulse.

Initialization String

You may wish to send some command to the modem to meet some

special need. The string you type in this box gets sent to the modem

just before dialing is done.  It is sent after the modem script has sent

its options.

SLIP (F5)

Configuration Window

A SLIP server is usually implemented with some form of security.

PHONE can be configured to respond correctly to your SLIP server's

security system by entering the proper values in the fields in this

window.

User Name

Enter your username as

username@computer.full.tcp.name

for example:

joe0024@student.tc.umn.edu

Password

Enter the password that goes with your username here.

It will not be displayed on the screen, so be sure to get it right.

Phone Number

Enter the phone number in this box.

As of Feb, 1993 some commonly used phone numbers (in Minnesota)

are:

% The Univ of Minn SLIP server for most modems is 626-1920.

% The Univ of Minn SLIP server when on-campus is 6-1920.

% The Univ of Minn SLIP server for ADI modems is 3-0291.

% The MRNET SLIP server is 626-7638.

If you are in a different area code, you'll need to prepend the area

code to these numbers.  In some area codes you also need to prepend

a 1.

Special Dialing Considerations

If you need to dial out from a hotel phone, or an office telephone

system, you may have to prepend a 8 or 9 to dial out of that local

phone system.  You may also have to add a comma to add a bit of

delay after the 8 or 9.  For example, to call the the U of M from a

hotel in California, you may have to specify:

8,1,612,626-1920

If you have call-waiting, your SLIP connection may occassionally be

interrupted by incoming telephone calls.  Depending on the service

available in your area, you may be able to suspend call-waiting for

the duration of one call.  This is accomplished with a special dialing

sequence which precedes the actual phone number

Tone		Pulse

*70		1170

To suspend call waiting from an off-campus telephone calling the

University of Minnesota, you would enter the phone number as

*70,6261920

or if using pulse dialing

1170,6261920

You may wish to test for this feature by hand before incorporating it

into your dialing sequence.

Server Window

A SLIP server is the remote computer which will be your gateway to

the TCP/IP network.  If more than one gateway exists, this window

will holds the list of gateways to choose from.  If only one gateway

exists, this window will not appear.  This list will vary from one local

network to another.

University of Minnesota students & staff should pick the UofM

server.

Time Display

For your convenience, the current time is displayed in the upper

right-hand corner of the screen.  This can be a useful device for

determining the timing of events if you happen to be writing a

custom script (see SCRIPTS below).

The Lower Status Bar

The lowest line on the screen is a status bar which will change to

offer you assistance with whatever portion of the program you

currently have active.  This bar has two parts:

Active Keys display

The left-hand portion shows active help keys highlighted in red, with

their names beside them in black.  For instance, at any time you can

press the F1 key to get Help with the current active item, so the

Active Keys display will usually show at least "F1 Help".

Context-Sensitive display

The right-hand portion of the Lower Status Bar indiciates the

purpose of the currently active item.  For instance, if the cursor is in

the Transaction Log window, the right-hand portion of the Lower

Status Bar reads "This window displays what is happening during a

login.", which is the function of the Transaction Log window.

Transaction Log Window

This window is where you can watch the interaction between your

computer and the remote computer which will provide SLIP service.

Labelled horizontal status bars indicate when important steps in the

login sequence are completed or fail to complete correctly.

Command-line operations

From the DOS command line, you can type

PHONE DIAL

to connect to the SLIP server, or

PHONE HANGUP

to disconnect from the SLIP server.  In some cases

PHONE FORCE <command>

where <command> is either DIAL or HANGUP may be necessary.  Try

these if PHONE is complaining that the modem is misbehaving.

PHONE WRITE

PHONE comes with several script files built in.  These include the

individual modem dialing scripts such as HAYES and ADI (see The

Primary Screen: Setup: MODEM above), plus the HANGUP command.

The PHONE WRITE command will extract these scripts into a file

called PHONE.CMD.  For more information, see the SCRIPTS section

below.

PHONE ?

Prints a brief description of the PHONE command-line options.

Scripts

Until now, using SLIP was difficult.  First you had to call a SLIP

modem pool, using a modem and a communications program such as

PROCOMM. Then you would tell the modem pool that you wanted to

use SLIP, and receive an IP address for the session.  Then you would

exit the communications program, being careful not to disconnect

your modem (by leaving DTR on for you experts), and edit a

configuration file, adding your IP address to the configuration.

Finally, you would load your packet driver and you could use Telnet

or FTP.

This was very cumbersome.

The PHONE program is designed to automatically connect to a local

SLIP server, configuring your computer and loading the UMSLIP

packet driver.  Ideally you can establish a SLIP connection simply by

using the command:

PHONE DIAL

from the DOS command line, or a batch file, and be connected

"automagically" .

Built-in scripts

To accomplish this task, PHONE understands a built-in scripting

language.  This lets you custom design a script to work at your

particular site.  You have to customize the script for each site for two

main reasons: (1) Each model of modem is slightly different and

needs different commands.  (2)  Each SLIPserver will have its own

custom prompt lines and will require different information for

logging on and entering SLIP mode.

We supply some generic scripts that work with Hayes-like modems

to dial our local U of MN SLIP server.  You can probably customize

this script for your particular site.  See the scripting language guide

which follows.

How to make custom scripts

PHONE comes with several built-in scripts.  However, these scripts

are not likely to work for anyone who is not at the University of

Minnesota.  Therefore, it will be necessary for many people to create

customized scripts.

PHONE WRITE

The first step in the process of creating customized scripts is to write

the built-in scripts out to a file.  It is often easiest to customize an

existing script rather than to start from scratch.  The PHONE WRITE

command writes the scripts out to the PHONE.CMD file.

PHONE.CMD

If the PHONE.CMD file is present in the directory with PHONE.EXE,

PHONE will read in PHONE.CMD rather than its built-in scripts.

With the SLIPScript language (see below), it's easy to create and use

customized SLIP scripts.

The SLIPScript Language

SLIPScript is a simple language for setting up the serial line, sending

characters, waiting for a character string.

Each SLIPScript script must contain at least two procedures, named

"Dial" and "Hangup".  The "Dial" script should contain commands to

open the SLIP connection.  The "Hangup" script should contain

commands to close the serial connection.

SLIPScript Elements

# comment

You can enter any text after a # character.  This text will be ignored

by the command processor.

The # comment can be on a line by itself, or appended to the end of

another command line.

command param1 param2 param3

SLIPScript commands consist of a keyword.  Valid command

keywords are:  Proc, EndProc, Send, Serial, Expect, DTR, Repeat, Flush,

TimeOut, Wait, and Set.

Some commands take parameters.  Parameters are separated by

spaces. If you need a space inside a parameter, enclose the whole

thing in quotes ("").

The TimeOut, Wait and Repeat commands take a time in seconds.

You can specify fractions of a second, i.e. 0.5 or 2.5 seconds.

SLIPScript commands

Procedure and EndProcedure

The "Proc" statement  identifies the beginning of a procedure.

Procedures continue until the next "EndProc" statement.

Example:

#

#     This is the shortest possible script

#

Procedure Modem.Hayes.Dial

EndProc   Modem.Hayes.Dial

The proc name has three parts, separated by dots.  The first part is

either "Modem" or "Host".  The second part is the name of the modem

or host.  The third part is either "Dial" or "Hangup" for a modem

script, or "Login" and "Logout" for a Host Script.

Join

The join command lets you continue execution in another script.  For

instance, many modems need some initial setup commands, then

they can join the standard Hayes modem script.  The join command

eliminates much duplicate scripting.

Serial

The "Serial" command lets you set up the serial port.  You can choose

which serial port to use, what baud rate to use, and choose even, odd,

or no parity.  You can also choose whether to use hardware

handshaking.  The parameters are:

COM port

Selects the COM port to use.

7 or 8

Selects 7 or 8 data bits.

9999

Any string of 3 or more digits means a baud rate.  Valid values are

300,1200, 2400, 4800, 9600, 19200, 38400, 57600.

E, O, N

Selects Even, Odd, or None parity.

hi+ or hi-

Sets input hardware handshaking on (+) or off (-).

ho+ or ho-

Sets output hardware handshaking on (+) or off (-).

Examples:

Serial Modem 1200		# simple 1200 baud modem

Serial Modem 38400 hi+ ho+	# fancy v32 modem

Notes:

Once established in SLIP mode, SLIP connections must be 8 data bits.

You may have to choose 7 data bits first to talk to the modem, PBX,

or the terminal server.

YouUll need hardware handshaking if you are using a modem with

data compression.

YouUll also need hardware handshaking if your modem auto-connects

at a different baud rate than the one it is talking to.  Otherwise it is

safer to use the default ( hi- and ho-).  See the section on Hardware

Handshaking for a discussion of hardware handshaking.

You may lose some incoming characters if they arrive just as the

Serial command is being executed.  It is best to place the serial

command at spots where you know is quiet.

Send

You use the Send statement to send out a stream of characters.

Anything you place on the same line after the "Send" keyword will

go out the serial line.  It is best to put the string in quotes ("") to

avoid ambiguity.

There are a few special characters that have special meaning:

< (less-than sign)

The less-than sign means send an ASCII "Return", or end of line

character.  For example, to get the attention of a Hayes-style modem:

Send AT<

%

The percent sign means to insert a variable in place of the % sign.

The character following the %  is a letter from a-z identifying the

variable.

Predefined variables include:

%n

for the phone number entered into the SLIP user dialog box.

%u

for the user name entered into the SLIP user dialog box.

%p

for the password entered into the SLIP user dialog box.

%s

for the custom init string entered into the SLIP modem dialog box.

^ (caret sign)

The up-caret lets you send any control character.  For instance you

can get the attention of a ADA 2020 modem by sending Control-Q:

Send ^Q

To send the caret character itself, send two carets together (e.g. ^^).

#

Remember that the # starts a comment.

You can define other variables with the"Set" command

There is a 1 second pause before each send command.  There is a

1/10 second pause between sending out each character, in order to

simulate typing speed.

SendDT

You use the SendDT statement to send the two forms of the dialing

command.  The first parameter is the pulse dialing command, the

second the tone dialing command.  For a Hayes modem, this is:

SendDT 'AT DP %n' 'AT DT %n'

Expect

The Expect statement has two parameters.  The first is a string of

characters to wait for.  The second is an error message to display if

the string is not seen.  The strings are not case-sensitive.

It waits for the default time of 10 seconds.  If it doesnt get the

specified string in that time, the script displays the error message,

quits execution and SLIP signals connection failure to the application.

Example:  Getting the ModemUs attention:

Send 'AT<'

Expect 'OK' 'Did not get OK'

Reject

The Reject statement detects error conditions. The Reject statement

has two parameters.  The first is a string of characters to wait for.

The second is an error message to display if the string is seen.  The

strings are not case-sensitive.

You can have any number of reject statements.  It waits for the

default time of 10 seconds.  If it does get the specified string in that

time, the script displays the error message, quits execution and SLIP

signals connection failure to the application.

If you dont specify an error message, the current "alert" string is

used.

Example:  Getting the ModemUs attention:

Send 'AT DT6-1920<'

Reject 'BUSY'          'Phone line is busy'

Reject 'NO CARRIER'    'No modem answered'

Expect 'OK'            'Did not get OK'

TimeOut

You can change the timeout interval for the next Expect statement

with the TimeOut command.   The TimeOut command takes two

parameters.  The first is the number of seconds to wait before timing

out.  The second parameter is an error message to display upon

timeout.

For example, if your phone system is slow to dial and connect, you

may want to say:

Send 'AT DT 636-1630<'

TimeOut 45  'Did not get connect in 45 seconds'

Expect 'CONNECT'

Repeat

The repeat command is useful when you have to retry sending a

string of characters to get attention.  For instance, many systems

require an indeterminate number of ASCII Return characters to

detect your baud rate.  The repeat command makes this easy.  You

can specify the number of repeats and the time interval between

repeats:

Example: to get the attention of a certain terminal server:

Repeat  '<'    'access:'  20 2.5

Will send "<"  which is the special character meaning ASCII Return,

until it receives the string"access:".  It will try up to 20 times, with a

2.5 second pause between attempts.

Or to get the attention of a certain modem that is slow to detect the

baud rate:

Repeat 'AT<' 'OK'  5 1.0

You need not specify the repeat count and pause time.  The default

repeat is 10 and default pause is 1.0 seconds.

Since the repeat command takes several parameters, you should

enclose strings in quotes to avoid any confusion as to which

parameter is which.

Example:

Repeat "<"  "Please log in now:"

Wait

The wait command pauses for the specified number of seconds.  This

is usually the wrong way to control interactions.  It is better to use

the "Expect" command to wait for a particular prompt.  But

sometimes a pause is the only way to meet some login requirement.

Example:  Calling out through a slow PBX

Send "Dial 8<"    	# Get an outside line

Wait 5.0			# Wait 5 sec for slow PBX

Send 'Dial %n'		# send the phone number

DTR

Some modems or PBXUs don't respond until the DTR line (RS-232 pin

20) is asserted.  The DTR command lets you control the DTR line.

Examples:

DTR On          	Asserts DTR

DTR Off		Drops DTR

DTR			Drops DTR, waits 1 second, asserts DTR

Note that many serial cables send the DTR line out pin 8, requiring a

special cable or adapter to move this to pin 20.  See Signals and

Cables above.

Flush

Sometimes you want to ignore any pending input characters as they

may be misleading.  The Flush command throws away any pending

input characters.  You often want to do this when you expect some

"logon banner" that might have some unpredictable text.  Flush that

stuff before you try to do an "Expect".

Example:

Send 'AT DT 636-1630<'

TimeOut 45

Expect 'CONNECT'	# wait for modems to connect

Flush

Repeat "<" "login:" 20 1.0   # hit return til login

Set

The set command lets you set up variables that you use again and

again.  You can set them up in one place instead of repeating them

over and over.   The syntax is:

Set x something

Sets the variable x to the value "something".  x can be any letter from

a to z.  Something can be anything including other % variable

references.

Example:  Your terminal server prompts with "Server #0001 for the

Main Campus:"

Set %h "Server ##0001 for the Main Campus:"

Send 'ATDT 636-1630'

Expect 'CONNECT'

Repeat '<' %h 20 2.0

Send 'terminal NoEdit<

Expect %h

Send 'terminal FlowControl None<'

Expect %h

Send 'terminal baudrate 9600<'

Expect %h

Send 'terminal databits 8<'

Expect %h

Writing a script

If possible, modify an existing script instead of writing a new one

from scratch.  The best way is to try connecting with a terminal

emulator, saving the output to a file.  This log file will guide you in

writing the script.

You'll have to note places where you had to pause for something to

respond.  Writing a script can be easy or difficult.  Many dial-up

ports a bit non-deterministic and you'll have to play with the Repeat

command until things work right.

When you write your script, make use of the variables the user has

set up in the PHONE menu, i.e. don't hard code "636-1630" as the

phone number, use the %p phone number variable.

Debugging a script

Try out scripts by choosing them in the Modem settings dialog box.

PHONE will run your Dial script.  It may fail  at first.  The most likely

failure is an Expect command failing.  If an "Expect" command fails, it

will show you what it expected and what it actually received.  Edit

the script and try again.

Remember to send a Return at the end of lines that expect a Return.

The Send command never sends a Return for automatically.

Alert

You can put up custom messages in case of error conditions.  With

the "Alert" command you can specify a message that will be put up if

an Expect command fails. e.g.:

Alert "Your modem may not be on?"

Send 'AT<'

Expect 'OK'

If PHONE doesnt get an "OK" in the specified time,  the user will see a

dialog box with "Your modem may not be on?".

Message

You can also put up an informative message to the user to keep them

appraised of how the dialing procedure is going.  Normally PHONE

displays each command as it is executed.  If you use the Message

command, that string will be displayed instead of the commands.

Your message will remain displayed until you change it with another

Message commmand, or until the script ends. e.g.:

Message "Getting the modem's attention"

Send 'AT<'

Expect 'OK'

Message "The modem is responding!"

Let us know about your scripts.

We will add other modem scripts to our list as time for testing

allows.  If you create a customized script which you think would be

useful to others, please feel free to send it to us at

slip@boombox.micro.umn.edu